# ENVIRONMENTAL COMPLIANCE GENERAL COMMENTS:

1. SPILLS – Contractor shall be responsible for all oil spills and equipment release incident containment, cleanup, and proper waste disposal.  All spills of Petroleum, Oils, Lubricants, and Chemicals shall be reported to the Division of Fire and Rescue Services (On Bethesda Campus (911), or (301) 435- 1908 (Fire and Emergency Services).

2. UNDERGROUND STORAGE TANKS (USTs’) – Underground Storage Tanks shall not be installed at NIH Owned or Leased Facilities.

3. RADIATION SAFETY – Questions concerning Radiation Safety at NIH Owned or Leased Facilities should be addressed to the Division of Radiation Safety (301) 496- 5774.

4. ALL CORRESPONDENCE, ENVIRONMENTAL PERMIT APPLICATIONS, ENVIRONMENTAL REGULATORY SUBMISSIONS, AND ENVIRONMENTAL REGULATORY DOCUMENTS REQUIRING SUBMISSION TO FEDERAL, STATE, AND/OR LOCAL REGULATORS, SHALL BE SUBMITTED THROUGH THE OFFICE OF RESEARCH FACILITIES (ORF), DIVISION OF ENVIRONMENTAL PROTECTION (DEP), UNLESS THE CONTRACTOR RECEIVES WRITTEN PERMISSION TO CONTACT AN ENVIRONMENTAL REGULATORY AGENCY DIRECTLY. (Note: Do not anticipate receiving permission to Contact an Environmental Regulatory Agency Directly)

5. CONSTRUCTION DUMPSTERS – For questions concerning Construction Dumpsters contact the Waste Resource Recovery Branch (WRRB) CDR Matthew Deptola, or Mike Stefan (301) 496 –7775.

# National Environmental Policy Act (NEPA) Compliance

Compliance Area – National Environmental Policy Act (NEPA) Compliance

Requirement - All projects must have completed NEPA documentation before groundbreaking

Actions Necessary - With DEP, complete NEPA Checklist and initiate preparation of any required documentation.

DEP Subject Matter Expert – DEP Mark Radke (301) 496-7775

# Affirmative Procurement - Recycled content

Compliance Area - Affirmative Procurement - Recycled content

Requirement – Purchase EPA designated products made with recycled content and other environmentally preferred materials unless justification is provided.

Actions Necessary – Evaluation of EPA comprehensive procurement Guidelines.

DEP Subject Matter Expert – Jaroslav Sebek (301) 496-7775

# Air Emissions

Compliance Area – Permitting

Requirement – New air emissions sources and major modifications of existing sources require Permits to Construct and/or Permits to Operate

Actions Necessary – Permit to Construct. Title V Permit to Operate. Amendments.

DEP Subject Matter Expert - Joe Musa (301) 496-7775

Compliance Area - Installation of Emergency Generators

Requirement – Large emergency generators require permits. Before permits can be requested, Maryland and some other states may require concurrence letter from Public Service Commission (PSC).

Actions Necessary – Permit to Construct. PSC Concurrence.

DEP Subject Matter Expert -Joe Musa (301) 496-7775

Compliance Area - New Source Review

Requirement – Certain larger scale new air emissions sources and major modifications of existing sources require New Source Review

Actions Necessary – New Source Review Evaluation Required. New Source Review Permit Required.

DEP Subject Matter Expert - Joe Musa (301) 496-7775

Compliance Area - Conformity

Requirement – New Air emission sources.

Actions Necessary – Record of Non-applicability and Conformity Evaluation and major modifications of existing sources that do not require New Source Review Permits require conformity evaluations and perhaps conformity determinations.

DEP Subject Matter Expert – Joe Musa (301) 496-7775

Compliance Area - Asbestos Containing Materials (ACM)

Requirement – Safety, health and environmental requirements apply to all projects involving potential ACM. Conduct Asbestos Assessment Review with DEP. Review with DEP.

Actions Necessary – Will this project involve removal of ACM or work in areas with ACM? If so, refer to DOSH. NESHAPS notification. Undetermined ACM (select unless definite proof). Meet DEP Requirements for shipment and disposal of ACM waste.

DEP Subject Matter Expert - Jake Deal 301-496-7775; (DOHS) John Veitch 301-496-2346;

# Cultural Resources

Compliance Area – Cultural Resources

Requirement – Does the project impact on protected archaeological sites? Consult with NIH Historic Preservation Officer.

Actions Necessary – Does the project impact on historic buildings? Consult with NIH Historic Preservation Officer.

DFP Subject Matter Expert - David Derenick (301) -435-1775

# Energy Conservation

Compliance Area – Energy Conservation

Requirement – All NIH facilities shall incorporate energy conservation features as required by E.O. 13123 and NIH Design Policy and Guidelines.

Actions Necessary – Comply with International Energy Conservation Code (IECC). Submit IECC plan review record at completion of Design Development Phase.

DFS Subject Matter Expert -Greg Leifer (301) 402- 2100

Compliance Area - Energy efficient products

Requirement – Design must encourage purchase and use of energy efficient products.

Actions Necessary – Review project documents for compliance. Submit waiver if energy efficient products cannot be used.

# Hazardous Materials (HAZMAT) Use

Compliance Area - Inventory

Requirement – Will the construction use HAZMAT on site? Include NIH Controlled Material Specifications. Federal, state and local requirements apply.

Actions Necessary – HAZMAT purchases and usage must be tracked following NIH Controlled Materials Specification (01548) and applicable regulations.

Compliance Area - Containment

Requirement – Requirements for bulk HAZMAT

Actions Necessary – Review with DEP

DEP Subject Matter Expert - Brian Kim (301) 496-7775

Compliance Area – Security

Requirement – HAZMAT in storage and transportation must meet security requirements.

Actions Necessary – Follow existing security plan. Use existing HAZMAT facilities. New security plan and/or facilities required.

Compliance Area - Explosives

Requirement – Requirements concerning security, transportation, receiving, storage and use of explosives in construction.

Actions Necessary – Secure applicable licenses and permits.

# Natural Resources

Compliance Area - Wetlands

Requirement – Does the project impact on wetlands areas?

Actions Necessary – Wetlands permit

DEP Subject Matter Expert – Brian Kim (301) 496-7775

Compliance Area - Reforestation

Requirement – If the project disturbs more than a state determined area of land reforestation requirements might apply. In Maryland the disturbed area limit is 40,000 Square Feet.

Actions Necessary – Reforestation required (MD = 15%)

DEP Subject Matter Expert – Ryan Marshall (301) 496-7775

Compliance Area - Landscaping

Requirement – Review plant list to screen for invasive species. Encourage use of native species.

Actions Necessary – DFS evaluation

# Storage Tanks for Petroleum Products and Other Hazardous Materials (HAZMAT)

Compliance Area - Installation of Above Ground Storage Tanks

Requirement – Requirements for locating, designing, permitting, installation, containment and operations apply to tanks used for storage of petroleum products or HAZMAT.

Actions Necessary – Submit tank system design plans to DEP.

DEP Subject Matter Expert – Daryl Moore (301) 496-7775

Compliance Area - Removal of Above Ground Storage Tanks

Requirement – Does this project involve removal of above tanks that contain or previously contained petroleum products or HAZMAT? Regulatory requirements for closure, removal, and disposal of tanks and contents must be met.

Actions Necessary – Submit plans for removal action

DEP Subject Matter Expert – Daryl Moore (301) 496-7775

Compliance Area - Installation of Underground Storage Tanks (UST)

Requirement – Does this project involve installation or modification of USTs? Design, permitting, installation and operational requirements apply to underground storage of petroleum products and HAZMAT.

Actions Necessary – Review with DEP. Submit plans for installation or modification of tank system prepared by PE. Schedule inspections of installation.

DEP Subject Matter Expert – Daryl Moore (301) 496-7775

Compliance Area - Removal of Underground storage tanks (UST)

Requirement – Does this project involve removal of USTs? Regulatory requirements for closure, removal, tank disposal and remediation of any contaminated soils must be met.

Actions Necessary – Review with DEP. Submit plans for removal by MDE-certified UST contractor.

DEP Subject Matter Expert – Daryl Moore (301) 496-7775

# Storm Water, Sediment and Erosion Control

Compliance Area - Sediment and Erosion Control (S&EC)

Requirement – S&EC must be considered in all projects involving soil disturbance. Maryland projects disturbing more than 5,000 square feet (or more than 100 cubic yards of soil) must be approved by the MDE for sediment and erosion control. Requirements in other states may vary.

Actions Necessary – DEP plan review disturbed area greater than 1,000 SF, less than 5,000 SF. Review and approval of plans by MDE if disturbed area greater than 5,000 SF or more than 100 cubic yards.

DEP Subject Matter Expert – Brian Kim (301) 496-7775

Compliance Area - Sediment and Erosion Control (S&EC)- Over 1 Acre

Requirement – Projects disturbing 1 acre or greater require an NPDES permit before groundbreaking can occur.

Actions Necessary – Review and approval of plans by MDE.

DEP Subject Matter Expert – Brian Kim (301) 496-7775

Compliance Area - Storm Water Management (SWM)

Requirement – SWM must be considered in all projects affecting runoff. Maryland projects disturbing more than 5,000 square feet must be approved by the MDE before groundbreaking can occur. Requirements in other states may vary.

Actions Necessary – DEP plan review. Approval of plans by MDE.

DEP Subject Matter Expert – Brian Kim (301) 496-7775

# Sustainability

Compliance Area - Sustainability

Requirement – For new facilities, project specific sustainability objectives must be established in the Basis of Design. Sustainability features must be selected in consideration of impacts and costs over life cycle.

Actions Necessary – Review with DEP. Include sustainability narrative and LEED checklist in Basis of Design. Sustainability Checklist.

DEP Subject Matter Expert – Jaroslav Sebek (301) 496-7775

Compliance Area - Sustainability Metrics

Requirement – Metrics for measuring attainment of sustainability goals should be established.

Actions Necessary – Review with DEP. LEED. Green Globes. SPIRIT (Projects on DoD property).

DEP Subject Matter Expert – Jaroslav Sebek (301) 496-7775

Compliance Area - Sustainability Goals

Requirement – Sustainability goals should be established for all projects. Goals are mandatory for all capital projects.

Actions Necessary – Guiding Principles. LEED Certified – Silver, Gold and Platinum; SPIRT Gold (Required for projects on DoD property)

DEP Subject Matter Expert – Jaroslav Sebek (301) 496-7775

# Waste Management and Recovery

Compliance Area - Chemical Waste (CW)

Requirement – Hazardous Wastes are an EPA/State regulated subset of wastes subject to NIH Chemical Waste (CW) requirements. [DEP Evaluation Needed]. Will the construction or demolition activity generate any Chemical Wastes on site? [CW Management Plan. Provide temporary accumulation areas for CW from construction. Design permanent accumulation and processing room for CW.] Will the completed project generate or accumulate Chemical Wastes on site? [EPA/State Hazardous Waste Generator Activity Form submission (new off campus buildings - not covered by campus ID]

Actions Necessary – See above

DEP Subject Matter Expert – (WRRB) CDR Matthew Deptola (301) 496-7775

Compliance Area - Medical Waste

Requirement – Will the facilities resulting from the completed project handle biohazardous agents or generate regulated medical wastes?

Actions Necessary – Review by DEP and DOSH. Plan on-site collection, storage, inactivation, decontamination and disposal capability. Plan shipping capability and off-site disposal.

DEP Subject Matter Expert – (WRRB) CDR Matthew Deptola (301) 496-7775

Compliance Area - Radioactive Waste

Requirement – Will the facilities resulting from the completed project handle radioactive materials generate radioactive wastes?

Actions Necessary – Plan review by DEP and DRS. NRC licensing requirements. On-site collection, storage capability. Radioactive waste processing and storage room. Shipping capability and off-site disposal.

Compliance Area - Solid Waste Recycling and Disposal (Construction Debris) Solid Waste

Requirement – Obtain services for management/recycling of non-hazardous solid wastes generated during construction project.

Actions Necessary – Review with DEP. Submit waste management and recycling plan. Use existing services.

DEP Subject Matter Expert – (WRRB) CDR Matthew Deptola (301) 496-7775

# Wastewater (Sanitary Sewer)

Compliance Area - Off-site Publicly Operated Treatment Works (POTW)

Requirement – New users, users with significant increases to wastewater discharges and users with new batch discharges may be required to obtain POTW approval or discharge permits.

Actions Necessary – DEP plan review. Submission and approval of plans by POTW. Preparation and submittal of Discharge Authorization Permit Application.

DEP Subject Matter Expert – Ryan Marshall (301) 496-7775

Compliance Area - On-site treatment systems

Requirement – New systems must meet design requirements and will require permits to construct. Requirements vary with size and type of system and jurisdiction. New users, users with significant increases to wastewater discharges and users with new batch discharges may be required to obtain owner approval.

Actions Necessary – DEP review. Engineered design. Permit to Construct. Permit to Operate. Submission and approval of plans by state or local jurisdiction.

DEP Subject Matter Expert – Ryan Marshall (301) 496-7775

# Water Supply

Compliance Area - Water Supply Assurance

Requirement – Confirm supply and authority to withdraw water if not from public source.

Actions Necessary – Certificate of service. Groundwater withdrawal permit. Surface water withdrawal permit (served by public system).

DEP Subject Matter Expert – Ryan Marshall 301-496-7775

Compliance Area - Safe Drinking Water Act

Requirement – Compliance with construction and operating requirements for public drinking water systems.

Actions Necessary – Permit to construct. Permit to operate.

DEP Subject Matter Expert – Ryan Marshall 301-496-7775

Compliance Area - Cross connection control

Requirement – Protect external supply and internal system from backflow`` and back siphonage from connections and uses. Cross connection control program may be required by supplier

Actions Necessary – DEP review of plans. Plan approval by supplier. Cross Connection Control Program.

DEP Subject Matter Expert – Ryan Marshall 301-496-7775

Compliance Area - Conservation

Requirement – All NIH facilities shall incorporate water conservation features as required by E.O. 13123 and NIH Design Requirements Manual.

Actions Necessary – Comply with water conservation requirements.

Compliance Area - Construction Dewatering

Requirement – Groundwater withdrawal restrictions for construction site dewatering

Actions Necessary – Apply for groundwater withdrawal permit.

DEP Subject Matter Expert – AJ Mueller 301-496-7775

Compliance Area - Well Abandonment

Requirement – Requirements for abandonment of wells

Actions Necessary – Comply with local requirements.

DEP Subject Matter Expert – Ryan Marshall and Daryl Moore 301-496-7775

Compliance Area - Well Drilling

Requirement – Construction requirements for new wells

Actions Necessary – Obtain well drilling permit.

DEP Subject Matter Expert – Ryan Marshall 301-496-7775

# Decontamination

Compliance Area - Area Clearance

Requirement – Removal of containers and hazardous materials owned by previous occupant from the work area. Confirm radiological and biohazardous agent decontamination.

Actions Necessary – Notify DOHS and DRS of areas impacted by project. Confirm clearance by DOHS and DRS (Signed stickers have been applied to doors of affected rooms).

DRS and DOHS Subject Matter Expert – (DOHS) 301-496-2346

Compliance Area - Infrastructure Assessment

Requirement – Assess intrinsic hazardous materials and contaminants in infrastructure of the affected area(s) of the facility.

Actions Necessary – DEP site observation visit (small jobs). Preliminary assessment (Phase I). Characterization assessment (Phase II). Combined assessment (Phase I and II) for laboratory areas.

DEP Subject Matter Expert – Jake Deal 301-496-7775

Compliance Area - Decontamination

Requirement – Removal and disposal hazardous substances present as contaminants and intrinsic materials.

Actions Necessary – Decontamination and remediation (Phase III)

DEP Subject Matter Expert – Jake Deal 301-496-7775

Compliance Area - Exit signs and smoke detectors

Requirement – These units may contain radioactive materials and must be disposed of following NRC requirements.

Actions Necessary – Prepare inventory of units in area decommissioned. Consult with Division of Radiation Safety (DRS). Remove units and deliver to DRS for disposal

# Closure of Regulated Areas

Compliance Area - EPA/State Regulated Hazardous Waste Accumulation Areas and Facilities

Requirement – Closure of hazardous waste accumulation areas, permitted facilities and other regulated units requires assessment and removal of contaminants. Formal notification and submission of a closure plan may be required.

Actions Necessary – Review with DEP. Conduct site assessment (Phases I and II). Prepare corrective action plan. Prepare formal closure plan. Final status survey and record.

DEP Subject Matter Expert – ECB Brian Kim (301) 496-7775 and WRRB Matt Deptola (301) 496-7775

Compliance Area - Polychlorinated Biphenyl (PCB) Sites

Requirement – Transformers, transformer vaults and other areas where PCBs have been used are subject to specific decontamination and disposal requirements. Changes in locations and removals of transformers and other PCB equipment may have to be reported to emergency responders. [ Consult with DEP]

Actions Necessary – Does the area to be decommissioned have PCB sites? [Retro fill and reclassify transformers as non- PCB. Remove PCB containing units. Assess PCB contamination. Prepare PCB remediation plan. Final status survey and record.]

DEP Subject Matter Expert – Brian Kim (301) 496-7775

Compliance Area - Radiation Areas

Requirement – Areas where use of NRC regulated radioactive materials (RAM) will be discontinued are subject to requirements for clearance and decontamination. Completion of a formal decommissioning protocol may be required. [Consult with Division of Radiation Safety]

Actions Necessary – Was RAM used in the areas to be decommissioned? [Obtain Division of Radiation Safety Clearance. Formal decommissioning protocol. Final status survey and record.]

# Miscellaneous Criteria

Compliance Area - Light Control

Requirement – Determine appropriate light control measures for project setting.

Actions Necessary – Set light control measures limits at property boundary.

Compliance Area - Noise Control

Requirement – Prevent excessive noise levels and adverse impacts on surrounding areas.

Actions Necessary – Set noise limits and work hours. Establish noise monitoring plan and locations.

Compliance Area - Pest Management

Requirement – Establish structural and operational requirements for pest control during deconstruction and after decommissioning.

Actions Necessary – Project review by pest control unit.